# NavalForge Concept

Preliminary conceptual design report

# 10 m patrol launch

Project: NF-DEMO-PATROL-10M | Revision: P1

Preliminary engineering result. It does not replace the responsible engineer, formal approval, validated software, detailed structural analysis or verified input data.

# Engineering gate

CONDITIONAL — preliminary results with active engineering reservations

# Main indicators

| Indicator | Value | Unit |
| --- | --- | --- |
| Displacement | 7,915.66 | kg |
| Maximum speed | 38.00 | kn |
| Required power | 299.13 | kW |
| Range | 510.29 | nmi |
| Corrected GM | 0.63 | m |
| Freeboard | 0.88 | m |
| Adherence | 100.00 | % |

# Requirement matrix

| ID | Requirement | Actual | Status |
| --- | --- | --- | --- |
| REQ10-LOA | Overall length shall not exceed 10.50 m | 10.0 m | PASS |
| REQ10-BEAM | Beam shall not exceed 3.30 m | 3.1 m | PASS |
| REQ10-SPEED | Maximum speed shall be at least 35 kn | 38.0 kn | PASS |
| REQ10-RANGE | Range shall be at least 170 nmi | 510.2880704456275 nmi | PASS |
| REQ10-GM | Preliminary corrected GM shall exceed 0.45 m | 0.6272697216930424 m | PASS |
| REQ10-FB | Preliminary freeboard shall exceed 0.50 m | 0.8802774910584624 m | PASS |
| REQ10-PAYLOAD | Payload shall be at least 500 kg | 500.0 kg | PASS |

# Active engineering reservations

GZ curve uses a wall-sided preliminary estimate; validate with inclined geometry before statutory use.

Partially filled tanks present; free-surface correction applied.

Consumption must be replaced by verified engine/propulsor curves for contractual use.

# Selected alternatives

## NF-ECO: NF-DEMO-PATROL-10M-V002

Lowest demonstrative cost among variants passing every mandatory requirement.

## NF-BALANCED: NF-DEMO-PATROL-10M-V004

Highest auditable multicriteria balance of cost, performance, range, GM and risk.

## NF-PERFORMANCE: NF-DEMO-PATROL-10M-V005

Highest combined performance and technical-margin screening among compliant variants.