# NavalForge Concept

Preliminary conceptual design report

# 12 m rescue and support craft

Project: NF-DEMO-RESCUE-12M | Revision: P1

Preliminary engineering result. It does not replace the responsible engineer, formal approval, validated software, detailed structural analysis or verified input data.

# Engineering gate

CONDITIONAL — preliminary results with active engineering reservations

# Main indicators

| Indicator | Value | Unit |
| --- | --- | --- |
| Displacement | 14,569.26 | kg |
| Maximum speed | 35.00 | kn |
| Required power | 382.64 | kW |
| Range | 557.56 | nmi |
| Corrected GM | 0.71 | m |
| Freeboard | 0.98 | m |
| Adherence | 100.00 | % |

# Requirement matrix

| ID | Requirement | Actual | Status |
| --- | --- | --- | --- |
| REQ12-LOA | Overall length shall not exceed 12.50 m | 12.0 m | PASS |
| REQ12-SPEED | Maximum speed shall be at least 32 kn | 35.0 kn | PASS |
| REQ12-RANGE | Range shall be at least 220 nmi | 557.5582257430633 nmi | PASS |
| REQ12-GM | Preliminary corrected GM shall exceed 0.50 m | 0.709702491315202 m | PASS |
| REQ12-FB | Preliminary freeboard shall exceed 0.60 m | 0.9772466208572445 m | PASS |
| REQ12-PAYLOAD | Payload shall be at least 1000 kg | 1000.0 kg | PASS |

# Active engineering reservations

GZ curve uses a wall-sided preliminary estimate; validate with inclined geometry before statutory use.

Partially filled tanks present; free-surface correction applied.

Consumption must be replaced by verified engine/propulsor curves for contractual use.

Speed is below the preferred planing-method range; displacement effects are simplified.

# Selected alternatives

## NF-ECO: NF-DEMO-RESCUE-12M-V001

Lowest demonstrative cost among variants passing every mandatory requirement.

## NF-BALANCED: NF-DEMO-RESCUE-12M-V009

Highest auditable multicriteria balance of cost, performance, range, GM and risk.

## NF-PERFORMANCE: NF-DEMO-RESCUE-12M-V003

Highest combined performance and technical-margin screening among compliant variants.