# NavalForge Concept

Preliminary conceptual design report

# 7 m service launch

Project: NF-DEMO-SERVICE-7M | Revision: P1

Preliminary engineering result. It does not replace the responsible engineer, formal approval, validated software, detailed structural analysis or verified input data.

# Engineering gate

CONDITIONAL — preliminary results with active engineering reservations

# Main indicators

| Indicator | Value | Unit |
| --- | --- | --- |
| Displacement | 2,675.47 | kg |
| Maximum speed | 34.00 | kn |
| Required power | 107.37 | kW |
| Range | 128.70 | nmi |
| Corrected GM | 0.48 | m |
| Freeboard | 0.75 | m |
| Adherence | 100.00 | % |

# Requirement matrix

| ID | Requirement | Actual | Status |
| --- | --- | --- | --- |
| REQ7-LOA | Overall length shall not exceed 7.50 m | 7.0 m | PASS |
| REQ7-SPEED | Maximum speed shall be at least 30 kn | 34.0 kn | PASS |
| REQ7-RANGE | Range shall be at least 75 nmi | 128.69700377961584 nmi | PASS |
| REQ7-GM | Preliminary corrected GM shall exceed 0.35 m | 0.47893410316254525 m | PASS |
| REQ7-FB | Preliminary freeboard shall exceed 0.40 m | 0.7469318304744539 m | PASS |
| REQ7-PAYLOAD | Payload shall be at least 300 kg | 300.0 kg | PASS |

# Active engineering reservations

GZ curve uses a wall-sided preliminary estimate; validate with inclined geometry before statutory use.

Partially filled tanks present; free-surface correction applied.

Consumption must be replaced by verified engine/propulsor curves for contractual use.

# Selected alternatives

## NF-ECO: NF-DEMO-SERVICE-7M-V002

Lowest demonstrative cost among variants passing every mandatory requirement.

## NF-BALANCED: NF-DEMO-SERVICE-7M-V005

Highest auditable multicriteria balance of cost, performance, range, GM and risk.

## NF-PERFORMANCE: NF-DEMO-SERVICE-7M-V007

Highest combined performance and technical-margin screening among compliant variants.